TRAINING REPORT

EMBEDDED C

| SUBMITTED BY: DEEPANSH MADAN |  |
| --- | --- |
| EMP ID:213458 |  |

Contents

1	TABLE OF FIGURES	3

2	MODULE DESIGN AND IMPLEMENTATION OF WIPER SYSTEM	4

2.1	CIRCUIT DESIGN	4

2.2	SIMULATION RESULT	4

https://www.tinkercad.com/things/diPfzxvVoTy-wiper-system-shiny-migelo/editel?sharecode=obhJcuppSEly7eiN8WKtTH4HIzR4fJXjlMp8T5j6Xa4=	4

2.3	EMBEDDED C CODE USING API	5

3	ILLUSTRATE RISC AND CISC ARCHITECTURE	7

3.1	RISC	7

3.2	CISC	7

4	BLOCK DIAGRAM OF COFFEE VENDING MACHINE ECU AND ITS DESIGN CHALLENGE CHART	8

4.1	BLOCK DIAGRAM	8

5	VON- NEUMAN AND HARVARD ARCHITECTURE	9

6	ASSOCIATIVITY OF OPERATORS IN C LANGUAGE	10

7	RGB interfacing on Tinkercad	11

7.1	Circuit Design	11

7.2	Simulation Result	11

7.3	Code	11

8	Size of Serial Registers in Atmega 328P	12

9	Frames in Ethernet	12

10	Prescalar	12

11	Simulator and Emulator	12

# TABLE OF FIGURES

Figure 1Wiper System Circuit	4

Figure 2Block Diagram of Coffee Vending Machine	8

Figure 3Design Challange Chart of Coffee Vending MachineDESIGN CHALLENGE CHART	8

Figure 4Associativity Table	10

Figure 5RGB Interfacing Circuit Diagram	11

# MODULE DESIGN AND IMPLEMENTATION OF WIPER SYSTEM

## CIRCUIT DESIGN

Figure 1Wiper System Circuit

## SIMULATION RESULT

https://www.tinkercad.com/things/diPfzxvVoTy-wiper-system-shiny-migelo/editel?sharecode=obhJcuppSEly7eiN8WKtTH4HIzR4fJXjlMp8T5j6Xa4=

## EMBEDDED C CODE USING API

#define HIGH 1

#define LOW 0

int ignPin =2;

int wipPin =4;

int rainPin = A0;

int ignLed = 13;

int motorOut=3;

int wipLed = 12;

void setup()

{

Serial.begin(9600);

pinMode(ignPin, INPUT);

pinMode(wipPin, INPUT);

pinMode(rainPin, INPUT);

pinMode(motorOut, OUTPUT);

pinMode(wipLed, OUTPUT);

pinMode(ignLed, OUTPUT);

digitalWrite (ignLed, LOW);

digitalWrite (wipLed, LOW);

analogWrite (motorOut,0);

}

int potent = 0;

int PWM =0;

void loop()

{

start:

while ((digitalRead(ignPin))== HIGH)				//ignition check

{

Serial.println("ignitionON");

digitalWrite(ignLed,HIGH);				//ignition LED

if(digitalRead (wipPin) == HIGH){				//wiper check

Serial.println("wiperON");

digitalWrite(wipLed,HIGH);

potent = analogRead(rainPin);				//rainsens input

if(potent >=5){						// IF RAIN >THRESHOLD

PWM = map(potent,0,1023,5,255);		// MAPPING FOR MOTOR

if (PWM >= 5 && PWM< 80){

analogWrite(motorOut,64);

}

else if(PWM>= 80 && PWM <160){

analogWrite(motorOut,191);

}

else if(PWM>= 160 && PWM <=255){

analogWrite(motorOut,255);

}

else {

Serial.println("ERROR... mapping 03");

goto start;

}

delay(5000);

Serial.println("fault 1");

goto start;

}

else if(potent <5){					// NO RAIN .. NO OUTPUT

analogWrite(motorOut, 0);

delay(5000);

goto start;

}

else {

Serial.println("ERROR.. rain detection 02");

goto start;

}

}

else if (digitalRead(wipPin)== LOW){

Serial.println("wiperOFF");

analogWrite(motorOut,0);

digitalWrite(wipLed , LOW);

goto start;

}

else{

Serial.println ("ERROR 01 ");

goto start;

}

}

Serial.println("ignitionOFF");

analogWrite(motorOut,0);

digitalWrite(ignLed, LOW);

digitalWrite(wipLed,LOW);

}

# ILLUSTRATE RISC AND CISC ARCHITECTURE

## RISC

RISC is an acronym for Reduced Instruction Set Code. Corresponds to basic hardware design where each small instruction is hard wired.

Instructions are simple and short, and take an average time of 1 to 1.5 clocks per instruction.

It has no memory unit to store complex instruction, complex software is written for high efficiency.

Less number of instruction set which can be combined to perform various complex tasks.

The architecture is heavily pipelined and multiple registers are present

The code can be decoded easily and requires no external memory for performing ALU tasks.

## CISC

CISC is an acronym for Complex Instruction Set Code. It performs multiple simpler instructions within a single instruction thus making it a complex instruction set.

Instructions are big and it takes an average of 2 to 15 clock cycles per instructions.

It stores big instructions in memory units and hence the complexity is on the hardware side.

Multiple instructions are present in different modes which can perform high level tasks very easily.

The architecture is less flexible, less pipelined and more focused on delivering the output of specific task.

# BLOCK DIAGRAM OF COFFEE VENDING MACHINE ECU AND ITS DESIGN CHALLENGE CHART

## BLOCK DIAGRAM

Figure 2Block Diagram of Coffee Vending Machine

Figure 3Design Challange Chart of Coffee Vending MachineDESIGN CHALLENGE CHART

# VON- NEUMAN AND HARVARD ARCHITECTURE

VON-NEUMAN ARCHITECTURE

Von- Neumann architecture is based on traditional design of storage of a software code, i.e. data and code is stored in a single physical memory in a continuous sequence.

Processor requires multiple cycles to fetch (mostly 1 instruction), decode(mostly 1) , execute(1 or more) and store(1 or 2 ) the instruction.

The number of cycles for each instruction is mostly same.

Only one permanent memory is used to store code and data. In some architecture code is stored in the starting address followed by its corresponding data.

While in others the starting address is a data (or vector address), which transfers the program control to initializing code.

The architecture is simple and costs less.

It has no pipelining and hence execution of instruction is slower but they are easy to handle.

HARVARD ARCHITECTURE

Harvard architecture or modern computer architecture is based on the concept of usage of different storage memories for data and code.

Processor requires less number of cycles to complete one instruction i.e. fetch, decode, execute, storage.

Steps like fetching, decode are processed in parallel or at same time, as data and code is stored at different places with separate buses for transfer of data

The number of cycles for each instruction depends on the complexity of pipelining.

Another important factor is efficient software code/program which reduces or omits bubbles in pipelining

Due to efficient management of pipelining, control unit and management of buses is complex which increases the cost.

APPLICATIONS

Von-Neumann Architecture

A Dataflow machine or state flow machine with less complex tasks and low memory usage or memory is used to access constants.

Harvard Architecture

Microcontrollers, DSP processors etc.

# ASSOCIATIVITY OF OPERATORS IN C LANGUAGE

Associativity of operators in C language is the rule which determines the flow of logic in absence of parenthesis. It can be of two types i) Right to Left ii) Left to Right

The following table shows the associativity of different operators.

Figure 4Associativity Table

Link for image: - https://www.oreilly.com/library/view/c11-for-programmers/9780133439885/graphics/08fig05.jpg

# RGB interfacing on Tinkercad

## Circuit Design

Figure 5RGB Interfacing Circuit Diagram

## Simulation Result

https://www.tinkercad.com/things/1Elit1VaRJw-fabulous-luulia-wluff/editel?sharecode=78sJc9UUlyLqJQZMwmnWinzkuEYCBiPOEMVR5YRwhNE=

## Code

void setup()

{

pinMode(9, OUTPUT);

pinMode(10, OUTPUT);

pinMode(11, OUTPUT);

pinMode(A0, INPUT);

pinMode(A1, INPUT);

pinMode(A2, INPUT);

}

void loop()

{

analogWrite(9 , ( analogRead(A0) / 4 ));

analogWrite(10 , ( analogRead(A1) / 4 ));

analogWrite(11 , ( analogRead(A2) / 4 ));

}

# Size of Serial Registers in Atmega 328P

The size is 1 byte or 2 byte.

# Frames in Ethernet

Frames of Ethernet range from 64 bytes to upto 1500 bytes

# Prescalar

A prescalar is a counting circuit which reduces the frequency of an input signal by dividing the frequency by a prescaled value which is a power of 2. They can be used in timers, counters, ADC modules or analog inputs. Major use of prescalar is to write delay subroutines or to reduce CPU clock frequency for certain applications.

# Simulator and Emulator

- Simulator

- Simulator works on the modeling of the system where each block is mimicked properly. The code runs through the proper methodology and presents the output.

- They can’t work with real time inputs

- They cannot be used for debugging.

- The host system cannot interact with the target system.

- Any changes in the hardware/target system cannot be reflected in simulation

- Emulators

- They do not copy the functionality of target system but rather interacts with the host system to give the output

- Majorly used for debugging.

- Real time inputs can be given.

- They use constant breakpoints to halt the processor and then copy its register, memory status values for displaying on the host system.

- Hence, they consume more  time than actual run time of processor.

- Changes in the hardware/ target system cause a change in the emulator output.